# Adding Writable Network Number Support to BACnet Virtual Router

This guide explains how to add writable Network_Number property support to both the BACnet IP network port object and virtual network port objects in a CAS BACnet Stack virtual router implementation. It assumes you have already implemented the basic virtual router with virtual devices.

## Overview

The Network_Number property (property identifier 425) on a Network Port object is optionally writable. When a BACnet client writes to it, the device must:

1. Store the pending new value.

2. Set Changes_Pending to true.

3. Wait for a ReinitializeDevice request to apply and broadcast the change. Three reinitialize states are relevant:

  - `ActivateChanges` – applies the pending changes at runtime with no restart.

  - `WarmStart` – applies the pending changes and performs a soft reboot of the device.

  - `ColdStart` – applies the pending changes and performs a hard reboot (full power-cycle restart) of the device. Behaviourally equivalent to WarmStart from the perspective of applying network port changes; both must be handled if the device supports the ReinitializeDevice service (required by BTL).

This behaviour applies to both the BACnet IP network port and to each virtual network port object that represents a virtual network.

## Step 1 – Extend the Database Model

### 1.1 Add prevNetworkNumber to the base network port class

The database needs to track the previous network number so that ActivateChanges can detect what changed and issue the correct BACnet broadcasts. Add prevNetworkNumber (and networkNumberQuality if not present) to ExampleDatabaseNetworkPortBase in `CASBACnetStackExampleDatabase.h`:

// CASBACnetStackExampleDatabase.h lines 55-62    
    class ExampleDatabaseNetworkPortBase : public ExampleDatabaseBaseObject    
    {    
    public:    
        bool     changesPending;    
        uint16_t networkNumber;    
        uint8_t  networkNumberQuality;  // unknown(0), learned(1), learned-configured(2), configured(3)    
        uint16_t prevNetworkNumber;    
    };

ExampleDatabaseNetworkPortIpv4 inherits from this base class and therefore automatically gains the same fields.

### 1.2 Initialise the fields in ExampleDatabase::Setup()

In `CASBACnetStackExampleDatabase.cpp`, make sure every field is initialised before use:

// CASBACnetStackExampleDatabase.cpp lines 49-54    
    // BACnet IP network port    
    this->networkPort.instance             = 1;    
    this->networkPort.objectName           = "Network Port for Ipv4";    
    this->networkPort.changesPending       = false;    
    this->networkPort.networkNumber        = 0;    
    this->networkPort.networkNumberQuality = 0;   // unknown – not yet configured    
    this->networkPort.prevNetworkNumber    = 0;

### 1.3 Initialise virtual network port fields in LoadVirtualDevices()

// CASBACnetStackExampleDatabase.cpp lines 62-69    
    ExampleDatabaseNetworkPortBase virtualNetworkPort;    
    virtualNetworkPort.changesPending       = false;    
    virtualNetworkPort.networkNumberQuality = 3;  // configured    
    virtualNetworkPort.instance             = STARTING_VIRTUAL_NETWORK + (networkIndex * VIRTUAL_NETWORK_OFFSET);    
    uint16_t networkNumber                  = STARTING_VIRTUAL_NETWORK + (networkIndex * VIRTUAL_NETWORK_OFFSET);    
    virtualNetworkPort.networkNumber        = networkNumber;    
    virtualNetworkPort.prevNetworkNumber    = networkNumber;  // matches current value – no pending change    
    this->virtualNetworkPorts[virtualNetworkPort.instance] = virtualNetworkPort;

## Step 2 – Enable WriteProperty on the Main Device

The WriteProperty service must be enabled on the main (router) device so that BACnet clients can write properties:

// BACnetVirtualDevicesServerExampleCPP.cpp lines 241-246    
    std::cout << "Enabling WriteProperty... ";    
    if (!fpSetServiceEnabled(g_database.mainDevice.instance,    
                             CASBACnetStackExampleConstants::SERVICE_WRITE_PROPERTY, true)) {    
        std::cerr << "Failed to enable the WriteProperty" << std::endl;    
        return false;    
    }    
    std::cout << "OK" << std::endl;

You also need ReinitializeDevice so that ActivateChanges / WarmStart can be sent:

// BACnetVirtualDevicesServerExampleCPP.cpp lines 248-253    
    std::cout << "Enabling ReinitializeDevice... ";    
    if (!fpSetServiceEnabled(g_database.mainDevice.instance,    
                             CASBACnetStackExampleConstants::SERVICE_REINITIALIZE_DEVICE, true)) {    
        std::cerr << "Failed to enable the ReinitializeDevice" << std::endl;    
        return false;    
    }    
    std::cout << "OK" << std::endl;

## Step 3 – Register the Set-Property Callback

Register the unsigned-integer write callback so the stack can deliver write requests for Network_Number:

// BACnetVirtualDevicesServerExampleCPP.cpp line 206    
    fpRegisterCallbackSetPropertyUnsignedInteger(CallbackSetPropertyUInt);

## Step 4 – Handle the Write in CallbackSetPropertyUInt

When the stack receives a WriteProperty for Network_Number it calls this callback. There are two cases to handle:

- BACnet IP network port – identified by objectInstance == g_database.networkPort.instance.

- Virtual network port – identified by a matching key in g_database.virtualNetworkPorts.

// BACnetVirtualDevicesServerExampleCPP.cpp lines 744-773    
    bool CallbackSetPropertyUInt(    
        const uint32_t deviceInstance,    
        const uint16_t objectType,    
        const uint32_t objectInstance,    
        const uint32_t propertyIdentifier,    
        const uint32_t value,    
        const bool     useArrayIndex,    
        const uint32_t propertyArrayIndex,    
        const uint8_t  priority,    
        uint32_t*      errorCode)    
    {    
        if (propertyIdentifier == CASBACnetStackExampleConstants::PROPERTY_IDENTIFIER_NETWORK_NUMBER) {    
            if (deviceInstance == g_database.mainDevice.instance) {    
         
                // Reject values outside the valid BACnet network number range    
                if (value > 65535) {    
                    *errorCode = CASBACnetStackExampleConstants::ERROR_VALUE_OUT_OF_RANGE;    
                    return false;    
                }    
         
                // --- BACnet IP network port ---    
                if (objectInstance == g_database.networkPort.instance) {    
                    if (objectType == CASBACnetStackExampleConstants::OBJECT_TYPE_NETWORK_PORT) {    
                        std::cout << "Received request to set Network Number property of the Network Port Object."    
                                  << " value=[" << value << "], priority=[" << (int)priority << "]" << std::endl;    
         
                        g_database.networkPort.prevNetworkNumber    = g_database.networkPort.networkNumber;    
                        g_database.networkPort.networkNumber        = value;    
                        g_database.networkPort.networkNumberQuality = 3;  // configured    
                        g_database.networkPort.changesPending       = true;    
                        return true;    
                    }    
                }    
         
                // --- Virtual network port ---    
                else if (g_database.virtualNetworkPorts.count(objectInstance) > 0) {    
                    g_database.virtualNetworkPorts[objectInstance].prevNetworkNumber    =    
                        g_database.virtualNetworkPorts[objectInstance].networkNumber;    
                    g_database.virtualNetworkPorts[objectInstance].networkNumber        = value;    
                    g_database.virtualNetworkPorts[objectInstance].networkNumberQuality = 3;    
                    g_database.virtualNetworkPorts[objectInstance].changesPending       = true;    
                    return true;    
                }    
            }    
        }    
        return false;    
    }

Key points

- Validate value <= 65535 first and return ERROR_VALUE_OUT_OF_RANGE if not — Network_Number is a Unsigned16 in the BACnet standard.

- Save the current value to prevNetworkNumber before overwriting networkNumber.

- Set networkNumberQuality to 3 (configured) to reflect that the value was explicitly set.

- Set changesPending = true. The stack reads this flag via CallbackGetPropertyBool and exposes it on the Changes_Pending property.

- Do not apply the change yet — that happens in ActivateChanges.

## Step 5 – Serve Changes_Pending via CallbackGetPropertyBool

The stack will call this callback when a client reads the Changes_Pending property. Handle both the BACnet IP port and virtual ports:

// BACnetVirtualDevicesServerExampleCPP.cpp lines 584-600    
    bool CallbackGetPropertyBool(    
        uint32_t  deviceInstance,    
        uint16_t  objectType,    
        uint32_t  objectInstance,    
        uint32_t  propertyIdentifier,    
        bool*     value,    
        bool      useArrayIndex,    
        uint32_t  propertyArrayIndex)    
    {    
        if (propertyIdentifier == CASBACnetStackExampleConstants::PROPERTY_IDENTIFIER_CHANGES_PENDING) {    
         
            // BACnet IP network port    
            if (objectType  == CASBACnetStackExampleConstants::OBJECT_TYPE_NETWORK_PORT &&    
                objectInstance == g_database.networkPort.instance) {    
                *value = g_database.networkPort.changesPending;    
                return true;    
            }    
         
            // Virtual network ports    
            if (objectType == CASBACnetStackExampleConstants::OBJECT_TYPE_NETWORK_PORT) {    
                if (g_database.virtualNetworkPorts.count(objectInstance) > 0) {    
                    *value = g_database.virtualNetworkPorts[objectInstance].changesPending;    
                    return true;    
                }    
            }    
        }    
        return false;    
    }

## Step 6 – Apply Changes in ActivateChanges()

ActivateChanges is called after ReinitializeDevice (ActivateChanges) is acknowledged. It must:

1. Detect which network numbers changed by comparing prevNetworkNumber with networkNumber.

2. Send a NetworkNumberIs broadcast for the BACnet IP port if its number changed.

3. Call fpUpdateVirtualNetworkNumber for each virtual port that changed, then send a single IAmRouterToNetwork broadcast.

// BACnetVirtualDevicesServerExampleCPP.cpp lines 391-431    
    void ActivateChanges() {    
        std::cout << "FYI: Activating Changes for NetworkPort Objects..." << std::endl;    
         
        // --- BACnet IP network port ---    
        if (g_database.networkPort.prevNetworkNumber != g_database.networkPort.networkNumber) {    
            std::cout << "Network Port network number changed. Sending NetworkNumberIs broadcast with new network number=["    
                      << g_database.networkPort.networkNumber << "]" << std::endl;    
            uint8_t connectionString[6];    
            memcpy(connectionString, g_database.networkPort.BroadcastIPAddress, 4);    
            connectionString[4] = g_database.networkPort.BACnetIPUDPPort / 256;    
            connectionString[5] = g_database.networkPort.BACnetIPUDPPort % 256;    
            if (!fpSendNetworkNumberIs(    
                    g_database.networkPort.networkNumber,    
                    g_database.networkPort.networkNumberQuality,    
                    connectionString, 6,    
                    CASBACnetStackExampleConstants::NETWORK_TYPE_IP,    
                    true, 65535, NULL, 0)) {    
                std::cerr << "Unable to send NetworkNumberIs broadcast for network number=["    
                          << g_database.networkPort.networkNumber << "]" << std::endl;    
            }    
            g_database.networkPort.prevNetworkNumber = g_database.networkPort.networkNumber;    
        }    
         
        // --- Virtual network ports ---    
        bool virtualNetworkPortChanged = false;    
        std::map<uint32_t, ExampleDatabaseNetworkPortBase>::iterator networkPortIt;    
        for (networkPortIt  = g_database.virtualNetworkPorts.begin();    
             networkPortIt != g_database.virtualNetworkPorts.end();    
             ++networkPortIt) {    
         
            if (networkPortIt->second.prevNetworkNumber != 0 &&    
                networkPortIt->second.prevNetworkNumber != networkPortIt->second.networkNumber) {    
         
                std::cout << "Virtual Network Port [" << networkPortIt->first    
                          << "] network number changed to ["    
                          << networkPortIt->second.networkNumber << "]" << std::endl;    
         
                virtualNetworkPortChanged = true;    
         
                if (!fpUpdateVirtualNetworkNumber(networkPortIt->second.prevNetworkNumber,    
                                                  networkPortIt->second.networkNumber)) {    
                    std::cerr << "Unable to update the virtual network number for virtual network port ["    
                              << networkPortIt->first << "]" << std::endl;    
                }    
                networkPortIt->second.prevNetworkNumber = networkPortIt->second.networkNumber;    
            }    
        }    
         
        if (virtualNetworkPortChanged) {    
            std::cout << "Sending IAmRouterToNetwork broadcast with new network numbers..." << std::endl;    
            uint8_t connectionString[6];    
            memcpy(connectionString, g_database.networkPort.BroadcastIPAddress, 4);    
            connectionString[4] = g_database.networkPort.BACnetIPUDPPort / 256;    
            connectionString[5] = g_database.networkPort.BACnetIPUDPPort % 256;    
            if (!fpSendIAmRouterToNetwork(connectionString, 6,    
                                          CASBACnetStackExampleConstants::NETWORK_TYPE_IP,    
                                          true, 65535, NULL, 0)) {    
                std::cerr << "Unable to send IAmRouterToNetwork broadcast" << std::endl;    
            }    
        }    
    }

Why `fpUpdateVirtualNetworkNumber`?
The CAS BACnet Stack internally routes messages using the virtual network number. When the number changes, the stack's routing table must be updated via this function before the new number is active.

## Step 7 – Handle ReinitializeDevice to Clear Changes_Pending

The ReinitializeDevice service supports three states that are relevant to network port changes:

| State | Meaning | Action |
| --- | --- | --- |
| ActivateChanges | Apply pending changes at runtime — no restart | Clear changesPending, signal deferred ActivateChanges() call |
| WarmStart | Apply pending changes then soft-reboot the device | Clear changesPending, signal deferred WarmStart() call |
| ColdStart | Apply pending changes then hard-reboot (power-cycle) the device | Same handling as WarmStart |

WarmStart and ColdStart are behaviourally equivalent from the perspective of applying network port changes — both apply pending changes and then restart the device. The difference is the type of restart: WarmStart is a software reboot and ColdStart is a full hardware power-cycle. This example does not handle `ColdStart`, but a production implementation must, as BTL requires a device to handle both WarmStart and ColdStart if it supports the ReinitializeDevice service.

In CallbackReinitializeDevice, clear changesPending for all network ports and set the appropriate flag so that ActivateChanges (or WarmStart) is processed outside the callback after the stack sends its SimpleAck:

// BACnetVirtualDevicesServerExampleCPP.cpp lines 775-836    
    bool CallbackReinitializeDevice(    
        const uint32_t  deviceInstance,    
        const uint32_t  reinitializedState,    
        const char*     password,    
        const uint32_t  passwordLength,    
        uint32_t*       errorCode)    
    {    
        // Validate password    
        if (password == NULL || passwordLength == 0) {    
            *errorCode = CASBACnetStackExampleConstants::ERROR_PASSWORD_FAILURE;    
            return false;    
        }    
        if (strncmp(password, "12345", passwordLength) != 0) {    
            *errorCode = CASBACnetStackExampleConstants::ERROR_PASSWORD_FAILURE;    
            return false;    
        }    
         
        if (reinitializedState == CASBACnetStackExampleConstants::REINITIALIZED_STATE_ACTIVATE_CHANGES) {    
         
            // Clear Changes_Pending on all network ports    
            g_database.networkPort.changesPending = false;    
            for (auto& kv : g_database.virtualNetworkPorts) {    
                kv.second.changesPending = false;    
            }    
         
            // Signal the main loop to call ActivateChanges() after the SimpleAck is sent    
            g_activateChanges = true;    
            return true;    
        }    
        else if (reinitializedState == CASBACnetStackExampleConstants::REINITIALIZED_STATE_WARM_START) {    
         
            g_database.networkPort.changesPending = false;    
            for (auto& kv : g_database.virtualNetworkPorts) {    
                kv.second.changesPending = false;    
            }    
         
            g_warmStart      = true;    
            g_warmStartTimer = time(0);    
            return true;    
        }    
        else {    
            *errorCode = CASBACnetStackExampleConstants::ERROR_OPTIONAL_FUNCTIONALITY_NOT_SUPPORTED;    
            return false;    
        }    
    }

Important: Never apply changes or restart the device directly inside the callback. Return true first so the stack can send the SimpleAck, then act on the flag in the main loop.

ColdStart not implemented: The else branch above returns ERROR_OPTIONAL_FUNCTIONALITY_NOT_SUPPORTED for all other states, including ColdStart. A BTL-compliant production device must add a ColdStart branch that mirrors the WarmStart handling (clear changesPending, set a deferred flag, then perform a full power-cycle restart after the SimpleAck is sent).

## Step 8 – Handle Deferred Activation in the Main Loop

The g_activateChanges and g_warmStart flags are consumed in the main loop:

// BACnetVirtualDevicesServerExampleCPP.cpp lines 153-162    
    for (;;) {    
        // Warm start (deferred 3 seconds to allow SimpleAck to be sent)    
        if (g_warmStart && g_warmStartTimer + 3 < time(0)) {    
            WarmStart();    
        }    
         
        // Activate changes    
        if (g_activateChanges) {    
            g_activateChanges = false;    
            ActivateChanges();    
        }    
         
        fpTick();    
        // ...    
    }

## Understanding ActivateChanges() and WarmStart()

This section walks through exactly what each function does in this example and highlights what a production implementation would need to add or change.

### ActivateChanges() – Apply pending changes at runtime (no restart)

ActivateChanges() (lines 391–431 in BACnetVirtualDevicesServerExampleCPP.cpp) is called from the main loop immediately after the stack has sent the SimpleAck for a ReinitializeDevice (ActivateChanges) request. It applies any pending Network_Number changes without restarting the device.

What this example does, step by step:

1. BACnet IP network port – Compares prevNetworkNumber with networkNumber. If they differ, builds the IP broadcast connection string inline, calls fpSendNetworkNumberIs() so every device on the BACnet/IP network learns the port's new network number, then resets prevNetworkNumber = networkNumber so a second call does not re-broadcast.

2. Virtual network ports – Iterates every entry in g_database.virtualNetworkPorts. For each port where prevNetworkNumber is non-zero and differs from networkNumber, calls fpUpdateVirtualNetworkNumber(prevNetworkNumber, networkNumber) to update the CAS BACnet Stack's internal routing table, sets a virtualNetworkPortChanged flag, then resets prevNetworkNumber = networkNumber.

3. `IAmRouterToNetwork` broadcast – If at least one virtual port changed, builds the broadcast connection string inline and sends a single fpSendIAmRouterToNetwork() so neighbouring routers and devices learn the updated network-to-router mapping.

What a production implementation must also do:

- Persist the new values – Write the updated network numbers to non-volatile storage (EEPROM, flash, config file, etc.) so they survive a power cycle. In this example, values are only held in RAM and are lost on restart.

### WarmStart() – Apply pending changes then soft-reboot

WarmStart() (lines 373–389 in BACnetVirtualDevicesServerExampleCPP.cpp) is called from the main loop after a 3-second deferred timer expires, giving the stack time to transmit the SimpleAck for the ReinitializeDevice (WarmStart) request before the restart begins.

What this example does, step by step:

1. `fpReset()` – Tears down and fully reinitialises the CAS BACnet Stack, clearing all registered objects, callbacks, and internal state.

2. `g_database.ReloadVirtualDevices()` – Clears the virtualDevices map and rebuilds it by iterating virtualNetworkPorts, using each port's current networkNumber value. This means any networkNumber that was written before the warm start is automatically picked up when the objects are re-registered in the next step.

3. `RegisterCallbacks()` – Re-registers all message, system-time, get-property, set-property, and remote-device-management callbacks with the freshly reset stack instance.

4. `SetupDevice()` – Re-adds the main device, network port object, virtual networks, virtual devices, and analog input objects to the stack. Returns false on failure, in which case WarmStart() logs an error and aborts.

5. `SendIAm()` – Broadcasts IAm, IAmRouterToNetwork, and NetworkNumberIs to re-announce the device and its updated routing information to the rest of the network. Returns false on failure, in which case WarmStart() logs an error and aborts.

What a production implementation must also do:

- Persist before resetting – Write all pending networkNumber values to non-volatile storage before calling fpReset(). In this example, ReloadVirtualDevices() reads from the in-RAM database (which still holds the written values), but on a real device the startup path typically reads from persistent storage, so values must be saved there first.

- `ColdStart` parity – A ColdStart handler must perform the same persistence step, but instead of calling fpReset() in-process it must trigger a full hardware power-cycle (e.g., a watchdog reset or a BSP restart call). The remaining steps — reload, re-register, re-setup, re-announce — then happen naturally as part of normal startup after the reboot.

## Step 9 – Send NetworkNumberIs on Startup

When the device starts up with a configured network number (quality = 3), broadcast a NetworkNumberIs so the rest of the network learns the mapping immediately:

// BACnetVirtualDevicesServerExampleCPP.cpp lines 362-368    
    // In SendIAm() – after sending IAmRouterToNetwork    
    if (g_database.networkPort.networkNumberQuality != 0) {    
        if (!fpSendNetworkNumberIs(    
                g_database.networkPort.networkNumber,    
                g_database.networkPort.networkNumberQuality,    
                connectionString, 6,    
                CASBACnetStackExampleConstants::NETWORK_TYPE_IP,    
                true, 65535, NULL, 0)) {    
            std::cerr << "Unable to send NetworkNumberIs broadcast for network number=["    
                      << g_database.networkPort.networkNumber << "]" << std::endl;    
            return false;    
        }    
    }

## Summary of Changes

| File | What changes |
| --- | --- |
| CASBACnetStackExampleDatabase.h | ExampleDatabaseNetworkPortBase gains prevNetworkNumber and networkNumberQuality fields |
| CASBACnetStackExampleDatabase.cpp | Setup() and LoadVirtualDevices() initialise the new fields, including prevNetworkNumber = 0 |
| BACnetVirtualDevicesServerExampleCPP.cpp | SetupDevice() enables WriteProperty and ReinitializeDevice; RegisterCallbacks() registers CallbackSetPropertyUInt; CallbackSetPropertyUInt handles PROPERTY_IDENTIFIER_NETWORK_NUMBER for both port types; CallbackGetPropertyBool serves Changes_Pending; CallbackReinitializeDevice clears flags and sets deferred-action globals; ActivateChanges() calls fpUpdateVirtualNetworkNumber and sends the appropriate broadcasts; SendIAm() sends NetworkNumberIs on startup |

## Property Flow Diagram

BACnet Client                     Your Server    
         |                                 |    
         |-- WriteProperty ---------------->|    
         |   Network_Number = 2001         |    
         |                                 | CallbackSetPropertyUInt:    
         |                                 |   prevNetworkNumber = old value    
         |                                 |   networkNumber     = 2001    
         |                                 |   changesPending    = true    
         |<-- SimpleAck -------------------|    
         |                                 |    
         |-- ReadProperty Changes_Pending ->|  CallbackGetPropertyBool → true    
         |<-- TRUE ------------------------|    
         |                                 |    
         |-- ReinitializeDevice ----------->|    
         |   (ActivateChanges, pw=12345)   |    
         |                                 | CallbackReinitializeDevice:    
         |                                 |   changesPending = false    
         |                                 |   g_activateChanges = true    
         |<-- SimpleAck -------------------|    
         |                                 |    
         |                                 | Main loop detects g_activateChanges:    
         |                                 |   fpUpdateVirtualNetworkNumber(old, 2001)    
         |                                 |   fpSendIAmRouterToNetwork(...)    
         |<-- IAmRouterToNetwork broadcast-|